1. OSI Layer of Wi-Fi

Wi-Fi operates in:

- Layer 1 – Physical Layer (PHY) → signal transmission

- Layer 2 – Data Link Layer (MAC) → medium access, framing

2. Daily Wi-Fi Devices + Capability Mapping

| Device | Band | Wi-Fi Standard | Features | Wi-Fi Generation |
| --- | --- | --- | --- | --- |
| Smartphone | 2.4/5 GHz | 802.11ac | MIMO | Wi-Fi 5 |
| Laptop | 2.4/5 GHz | 802.11ax | OFDMA, MU-MIMO | Wi-Fi 6 |
| Smart TV | 2.4 GHz | 802.11n | MIMO | Wi-Fi 4 |

3. BSS and ESS

- BSS (Basic Service Set) → One AP + clients

- ESS (Extended Service Set) → Multiple BSS with same SSID

4. Basic Functionalities of Access Point

- Beacon transmission

- Authentication & association

- Security

- Traffic forwarding

- QoS handling

- Power management support

5. Bridge Mode vs Repeater Mode

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Purpose | Connect networks | Extend coverage |
| Data flow | Network ↔ Network | AP → Repeater → Client |
| Use case | Building link | Range extension |

6. 802.11a vs 802.11b

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Frequency | 5 GHz | 2.4 GHz |
| Speed | Up to 54 Mbps | Up to 11 Mbps |
| Interference | Low | High |
| Range | Short | Longer |

7. 2.4 GHz vs 5 GHz Comparison (Experiment)

| Feature | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Range | High | Low |
| Speed | Lower | Higher |
| Interference | High | Low |
| Channels | Few | Many |

8. IEEE vs WFA

| Feature | IEEE | WFA (Wi-Fi Alliance) |
| --- | --- | --- |
| Role | Develop standards | Certify devices |
| Example | 802.11ax | Wi-Fi 6 certification |

9. Wi-Fi Backhaul Types

- Fiber

- Ethernet

- MoCA

- Powerline

- Cellular

- Satellite

10. Wi-Fi Topologies and Use Cases

| Topology | Description | Use Case |
| --- | --- | --- |
| Infrastructure | AP-based network | Homes, offices |
| Ad-hoc | Direct connection | Temporary sharing |
| Mesh | Multi-AP network | Large coverage |
| Bridge | Network connection | Building links |
| Repeater | Range extension | Weak signal areas |