1. Different 802.11 PHY layer standards and comparison

| Standard | Band | Technique | Max Rate |
| --- | --- | --- | --- |
| 11b | 2.4 GHz | DSSS | 11 Mbps |
| 11a/g | 5 / 2.4 GHz | OFDM | 54 Mbps |
| 11n | 2.4/5 | OFDM + MIMO | 600 Mbps |
| 11ac | 5 | OFDM + MU-MIMO | Gbps |
| 11ax | 2.4/5/6 | OFDMA | Higher |

2. DSSS and FHSS

DSSS (Direct Sequence Spread Spectrum):

- Each bit is multiplied by a pseudo-random code

- Spreads signal over wide bandwidth

- Receiver uses same code to recover data

FHSS (Frequency Hopping Spread Spectrum):

- Signal rapidly switches frequencies

- Uses predefined hopping pattern

- Receiver follows same pattern

3. Modulation schemes in PHY layer

Modulation converts bits into signals.

Common schemes:

- BPSK → 1 bit/symbol

- QPSK → 2 bits/symbol

- 16-QAM → 4 bits/symbol

- 64-QAM → 6 bits/symbol

- 256-QAM → 8 bits/symbol

| Scheme | Bits/Symbol | Speed | Noise Resistance |
| --- | --- | --- | --- |
| BPSK | 1 | Low | Very high |
| QPSK | 2 | Medium | High |
| QAM | 4–8 | High | Lower |

4. Significance of OFDM in WLAN

OFDM (Orthogonal Frequency Division Multiplexing):

- Splits data into multiple subcarriers

- Transmits in parallel

Advantages:

- Reduces multipath interference

- Improves spectral efficiency

- Enables high data rates

5. Frequency bands in Wi-Fi

Wi-Fi operates in:

- 2.4 GHz band

  - Fewer channels

  - Better range

- 5 GHz band

  - More channels

  - Higher speed, less interference

- 6 GHz band (Wi-Fi 6E)

  - Very wide bandwidth

  - Less congestion

Channels are divided into smaller frequency blocks used for transmission.

6. Guard Interval (GI)

Guard Interval:

- Time gap between OFDM symbols

- Prevents inter-symbol interference (ISI)

Short GI:

- Reduces idle time

- Increases data rate

- Requires good channel conditions

7. Structure of 802.11 PHY frame

PHY frame consists of:

- Preamble

  - L-STF, L-LTF (synchronization, channel estimation)

- Header

  - L-SIG (rate, length)

- Payload

  - Actual data

Flow:
Preamble → Header → Data

8. OFDM vs OFDMA

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Users | Single | Multiple |
| Allocation | Entire bandwidth | Subcarriers divided |
| Efficiency | Lower | Higher |

OFDMA allows multiple users to transmit simultaneously.

9. MIMO vs MU-MIMO

| Feature | MIMO | MU-MIMO |
| --- | --- | --- |
| Users | Single user | Multiple users |
| Streams | Multiple streams | Shared streams |
| Benefit | Higher speed | Multi-user efficiency |

10. PPDU, PLCP, PMD

- PLCP (Physical Layer Convergence Protocol)

  - Converts MAC data to PHY format

- PMD (Physical Medium Dependent)

  - Handles actual transmission

- PPDU (PLCP Protocol Data Unit)

  - Final transmitted PHY packet

11. Types of PPDU

Types depend on Wi-Fi generation:

- Legacy PPDU (802.11a/g)

- HT PPDU (802.11n)

- VHT PPDU (802.11ac)

- HE PPDU (802.11ax)

General structure:

- Preamble (training fields)

- Signal fields

- Data field

Each newer version adds more fields for higher efficiency and multi-user support.

12. Data rate calculation

Data rate depends on:

- Modulation (bits per symbol)

- Coding rate

- Number of subcarriers

- Symbol duration

- Number of spatial streams

General formula:

Data Rate =
(Number of bits per symbol × Number of subcarriers × Coding rate × Number of spatial streams) / Symbol duration

Higher modulation, more streams, and shorter guard interval increase data rate.