1. Significance of MAC Layer & OSI Position

- MAC (Medium Access Control) is part of Layer 2 (Data Link Layer) in OSI

- It controls:

  - Access to wireless medium

  - Frame transmission

  - Addressing (MAC address)

Importance:

- Prevents collisions (CSMA/CA)

- Ensures reliable delivery (ACK, retransmission)

- Handles QoS, power saving, aggregation

2. 802.11 MAC Frame Format

Structure:

- FCS (error check)

- MAC Header

- Frame Body

3. MAC Layer Functionalities

Management

- Scanning (active/passive)

- Authentication

- Association / Reassociation

- Beacon transmission

Control

- RTS/CTS

- ACK

- Block ACK

- PS-Poll

Data

- Frame transmission

- Aggregation (A-MSDU, A-MPDU)

4. Scanning Process

Types:

Passive Scanning

- AP sends Beacon

- Client listens

Active Scanning

- Client sends Probe Request

- AP replies with Probe Response

5. Client Association Process

- Scanning

- Authentication

- Association Request

- Association Response

- Data transfer begins

- EAPOL 4-Way Handshake (Extensible Authentication Protocol over LAN)

Purpose:

- Establish encryption keys

Steps:

- AP -> Client (ANonce)

- Client -> AP (SNonce + MIC)

- AP -> Client (GTK)

- Client -> AP (ACK)

Keys:

- PTK -> unicast encryption

- GTK -> multicast/broadcast

7. Power Saving Mechanisms

Power management schemes place a client in sleep mode when no activity occurs

• The MS can be configured to be in continuous aware mode (CAM) or Power Save Polling (PSP) mode

• In the PSP mode, the client can go to sleep by informing the AP when there is no activity

• The APs buffers any data directed to the client when the client is asleep

8. Medium Access Control Methods

- DCF -> random access (CSMA/CA)

- PCF -> polling

- EDCA -> QoS-based priority

9. Block ACK Mechanism

Working:

- Multiple frames sent

- One ACK with bitmap

- Retransmit only missing frames

Advantages:

- Reduces overhead

- Improves throughput

- Works with aggregation

10. A-MSDU, A-MPDU, A-MSDU in A-MPDU

A-MSDU

- Combine MSDUs into one frame

- One MAC header

- Less overhead, less reliable

A-MPDU

- Combine multiple MPDUs

- Each has its own header + FCS

- More reliable (partial retransmission)

A-MSDU inside A-MPDU

- Combine both methods

- High efficiency + high reliability